| Dip sw.1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coin mode 1 |  |  |  |  | Off |  |  |  |
| Coin mode 2 |  |  |  |  | On |  |  |  |
| Mode 1 |  |  |  |  |  |  |  |  |
| 1coin 1play | Off | Off | Off | Off |  |  |  |  |
| 2coin 1play | On | Off | Off | Off |  |  |  |  |
| 3coin 1play | Off | On | Off | Off |  |  |  |  |
| 4coin 1play | On | On | Off | Off |  |  |  |  |
| 5coin 1play | Off | Off | On | Off |  |  |  |  |
| 6coin 1play | On | Off | On | Off |  |  |  |  |
| 1coin 2play | Off | On | On | Off |  |  |  |  |
| 1coin 3play | On | On | On | Off |  |  |  |  |
| 2coin 4play | Off | Off | Off | On |  |  |  |  |
| 2coin 5play | On | Off | Off | On |  |  |  |  |
| 1coin 6play | Off | On | On | On |  |  |  |  |
| 2coin 3play | On | On | On | On |  |  |  |  |
| 3coin 2play | Off | Off | Off | On |  |  |  |  |
| 5coin 3play | On | Off | Off | On |  |  |  |  |
| 8coin 3play | Off | On | On | On |  |  |  |  |
| Free play | On | On | On | On |  |  |  |  |
| Mode 2 |  |  |  |  |  |  |  |  |
| 1coin 1play | Off | Off |  |  |  |  |  |  |
| 2coin 1play | On | Off |  |  |  |  |  |  |
| 3coin 1play | Off | On |  |  |  |  |  |  |
| 5coin 1play | On | On |  |  |  |  |  |  |
| 1coin 2play |  |  | Off | Off |  |  |  |  |
| 1coin 3play |  |  | On | Off |  |  |  |  |
| 1coin 5play |  |  | Off | On |  |  |  |  |
| 1coin 6play |  |  | On | On |  |  |  |  |
| Always |  |  |  |  |  | Off | Off |  |
| Track ball small |  |  |  |  |  |  |  | Off |
| Track ball large |  |  |  |  |  |  |  | On |
| Dip sw.2 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 3 lives | Off | Off |  |  |  |  |  |  |
| 2 lives | On | Off |  |  |  |  |  |  |
| 5 lives | Off | On |  |  |  |  |  |  |
| Unlimited | On | On |  |  |  |  |  |  |
| 20k-50k |  |  | Off | Off |  |  |  |  |
| 30k-100k |  |  | On | Off |  |  |  |  |
| 50k-150k |  |  | Off | On |  |  |  |  |
| 70k |  |  | On | On |  |  |  |  |
| Level-1 |  |  |  |  | Off | Off |  |  |
| Level-2 |  |  |  |  | On | Off |  |  |
| Level-3 |  |  |  |  | Off | On |  |  |
| Level-4 |  |  |  |  | On | On |  |  |
| Demo sound on |  |  |  |  |  |  | Off | Off |
| Demo sound off |  |  |  |  |  |  | Off | On |