| Parts side |  | Solder side |  | Parts side |  | Solder side |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Video red | 1 | Video green |  | Ground | 1 | Ground |  |  |
| Video blue | 2 | Video sync |  | Ground | 2 | Ground |  |  |
| Speaker | 3 | Speaker |  | +5 volt | 3 | +5 volt |  |  |
|  | 4 |  |  | +5 volt | 4 | +5 volt |  |  |
|  | 5 |  |  | +12 volt | 5 | +12 volt |  |  |
|  | 6 |  |  | Counter | 6 |  |  |  |
|  | 7 |  |  | Hopper main | 7 | Hopper main |  |  |
| Service coin | 8 |  |  |  | 8 |  |  |  |
| Start | 9 |  |  | Ground | 9 | Ground |  |  |
| Odds/small | 10 |  |  | Ground | 0 | Ground |  |  |
| Bet | 11 |  |  |  |  |  |  |  |
| Take score | 12 |  |  |  |  |  |  |  |
| Double up | 13 |  |  |  |  |  |  |  |
|  | 14 |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |  |
| Big | 16 |  |  |  |  |  |  |  |
|  | 17 |  |  |  |  |  |  |  |
| Coin A | 18 | Key in |  |  |  |  |  |  |
| Coin C | 19 | Coin D |  |  |  |  |  |  |
| Book | 20 | Analize |  |  |  |  |  |  |
| Pay out | 21 | Key down |  |  |  |  |  |  |
| Hopper full (to ground) | 22 | Hopper sw. |  |  |  |  |  |  |
| Counter A | 23 |  |  |  |  |  |  |  |
| Counter B (key in) | 24 |  |  |  |  |  |  |  |
| Counter C | 25 |  |  |  |  |  |  |  |
| Counter D (hopper) | 26 |  |  |  |  |  |  |  |
| Hopper counter | 27 | Lack of hopper counter |  |  |  |  |  |  |
| Key down counter | 28 |  |  |  |  |  |  |  |
| Start lamp | 29 |  |  |  |  |  |  |  |
| Cancel lamp | 30 |  |  |  |  |  |  |  |
| Bet lamp | 31 |  |  |  |  |  |  |  |
| Take score lamp | 32 |  |  |  |  |  |  |  |
| Double up lamp | 33 |  |  |  |  |  |  |  |
|  | 34 |  |  |  |  |  |  |  |
|  | 35 |  |  |  |  |  |  |  |
| Ground | 36 | Ground |  |  |  |  |  |  |