| DIP SW.1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal screen | Off |  |  |  |  |  |  |  |
| Flip screen | On |  |  |  |  |  |  |  |
| Second bonus |  |  | Off |  |  |  |  |  |
| Every bonus |  |  | On |  |  |  |  |  |
| 2 player |  |  |  | Off |  |  |  |  |
| 3 player |  |  |  | On |  |  |  |  |
| Coin 1 |  |  |  |  |  |  |  |  |
| 1coin 1play |  |  |  |  | Off | Off |  |  |
| 2coin 1play |  |  |  |  | On | Off |  |  |
| 3coin 1play |  |  |  |  | Off | On |  |  |
| 4coin 1play |  |  |  |  | On | On |  |  |
| Coin 1 |  |  |  |  |  |  |  |  |
| 1coin 1play |  |  |  |  |  |  | Off | Off |
| 1coin 2play |  |  |  |  |  |  | On | Off |
| 1coin 3play |  |  |  |  |  |  | Off | On |
| 1coin 4play |  |  |  |  |  |  | On | On |
| Always |  | Off |  |  |  |  |  |  |
| DIP SW.2 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Normal | Off | Off |  |  |  |  |  |  |
| Easy | On | Off |  |  |  |  |  |  |
| Hard | Off | On |  |  |  |  |  |  |
| Very hard | On | On |  |  |  |  |  |  |
| Demo sound on |  |  | Off | Off |  |  |  |  |
| Demo sound off |  |  | On | Off |  |  |  |  |
| Never finish |  |  | Off | On |  |  |  |  |
| Stop video display |  |  | On | On |  |  |  |  |
| 20000/50000 |  |  |  |  | Off | Off |  |  |
| 40000/100000 |  |  |  |  | On | Off |  |  |
| 50000/150000 |  |  |  |  | Off | On |  |  |
| No bonus |  |  |  |  | On | On |  |  |
| Continue on |  |  |  |  |  |  | Off |  |
| Continue off |  |  |  |  |  |  | On |  |
| Always |  |  |  |  |  |  |  | Off |