| DIP SW.A | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Table | Off |  |  |  |  |  |  |  |
| Upright | On |  |  |  |  |  |  |  |
| Normal picture |  | Off |  |  |  |  |  |  |
| Reverse picture |  | On |  |  |  |  |  |  |
| Game mode |  |  | Off |  |  |  |  |  |
| Test mode |  |  | On |  |  |  |  |  |
| Demo sound on |  |  |  | Off |  |  |  |  |
| Demo sound off |  |  |  | On |  |  |  |  |
| 1coin 1play |  |  |  |  | Off | Off |  |  |
| 1coin 2play |  |  |  |  | On | Off |  |  |
| 2coin 1play |  |  |  |  | Off | On |  |  |
| 2coin 3play |  |  |  |  | On | On |  |  |
| Always |  |  |  |  |  |  | Off | Off |
| DIP SW.B | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Normal | Off | Off |  |  |  |  |  |  |
| Easy | On | Off |  |  |  |  |  |  |
| Hard | Off | On |  |  |  |  |  |  |
| Very hard | On | On |  |  |  |  |  |  |
| 2 replay |  |  | Off | Off |  |  |  |  |
| 3 replay |  |  | On | Off |  |  |  |  |
| 1 replay |  |  | Off | On |  |  |  |  |
| 0 replay |  |  | On | On |  |  |  |  |
| Bomb off |  |  |  |  | Off |  |  |  |
| Bomb on |  |  |  |  | On |  |  |  |
| Girl on |  |  |  |  |  | Off |  |  |
| Girl off |  |  |  |  |  | On |  |  |
| Always |  |  |  |  |  |  | Off | Off |