| Dip sw 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal screen | Off | Off |  | Off |  |  |  |  |
| Reverse screen | On | Off |  |  |  |  |  |  |
| 2nd bonus |  |  | Off |  |  |  |  |  |
| Every bonus |  |  | On |  |  |  |  |  |
| 1coin 1play |  |  |  |  | Off | Off |  |  |
| 2coin 1play |  |  |  |  | On | Off |  |  |
| 3coin 1play |  |  |  |  | Off | On |  |  |
| 4coin 1play |  |  |  |  | On | On |  |  |
| 1coin 2play |  |  |  |  | Off | Off |  |  |
| 1coin 3play |  |  |  |  | On | Off |  |  |
| 1coin 5play |  |  |  |  | Off | On |  |  |
| 1coin 6play |  |  |  |  | On | On |  |  |
| 2 lives |  |  |  |  |  |  | Off | Off |
| 1 life |  |  |  |  |  |  | On | Off |
| 3 lives |  |  |  |  |  |  | Off | On |
| 4 lives |  |  |  |  |  |  | On | On |
| Dip sw 2 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Normal | Off | Off |  |  |  |  |  |  |
| Easy | On | Off |  |  |  |  |  |  |
| Hard | Off | On |  |  |  |  |  |  |
| Very hard | On | On |  |  |  |  |  |  |
| Demo sound on |  |  | Off | Off |  |  |  |  |
| Demo sound off |  |  | On | Off |  |  |  |  |
| Never finish |  |  | Off | On |  |  |  |  |
| Stop video display |  |  | On | On |  |  |  |  |
| 200k/400k |  |  |  |  | Off | Off |  |  |
| 400k/600k |  |  |  |  | On | Off |  |  |
| 600k/800k |  |  |  |  | Off | On |  |  |
| None |  |  |  |  | On | On |  |  |
| Continue on |  |  |  |  |  |  | Off | Off |
| Continue off |  |  |  |  |  |  | On | Off |